- Make your Arduino board blink an LED.

Aim:

To make a LED Blink operation using Arduino

Procedure:

- Start the Arduino IDE and enter the code

- Declare the variable for input pin

- Initialize digital pin 7 as an output in void setup function

- The loop function runs over and over again forever

- Turn the LED on by making the voltage HIGH

- Turn the LED on by making the voltage LOW

Program

int led = 7;

void setup()

{

pinMode(led, OUTPUT);

}

void loop()

{

digitalWrite(led, HIGH);

delay(1000);

digitalWrite(led, LOW);

delay(1000);

}

- Create an application to find Motion detection using arduino board.

Aim:

To find Motion detection using Arduino board.

Procedure:

- Start the Arduino IDE and enter the code

- Declare the PIR sensor output variable

- Define the baud rate for serial transmission

- The loop function runs over and over again forever

- Based on the sensor output show the movement.

- If the PIR sensor value is 1, then print the statement of movement detection

- If the PIR sensor value is 0, then print the statement of no movement detection.

Program:

int led = 13;                // the pin that the LED is atteched to

int sensor = 2;              // the pin that the sensor is atteched to

int state = LOW;             // by default, no motion detected

int val = 0;                 // variable to store the sensor status (value)

void setup() {

pinMode(led, OUTPUT);      // initalize LED as an output

pinMode(sensor, INPUT);    // initialize sensor as an input

Serial.begin(9600);        // initialize serial

}

void loop(){

val = digitalRead(sensor);   // read sensor value

if (val == HIGH) {           // check if the sensor is HIGH

digitalWrite(led, HIGH);   // turn LED ON

delay(500);                // delay 100 milliseconds

if (state == LOW) {

Serial.println("Motion detected!");

state = HIGH;       // update variable state to HIGH

}

}

else {

digitalWrite(led, LOW); // turn LED OFF

delay(500);             // delay 200 milliseconds

if (state == HIGH){

Serial.println("Motion stopped!");

state = LOW;       // update variable state to LOW

}

}

}

- Create an application to measure Temperature and Humidity using arduino board.

Aim:

To find measure temperature and humidity using Arduino board.

Procedure:

- Start the Arduino IDE and enter the code

- Declare the libraries for DHT11 sensor

- Define the baud rate for serial transmission

- The loop function runs over and over again forever

- Based on the sensor output show the current temperature and humidity.

- Display the temperature and humidity in centigrade and Fahrenheit values in serial monitor.

Program:

#include "DHT.h"

#define DHTPIN 2     // Digital pin connected to the DHT sensor

#define DHTTYPE DHT11   // DHT 11

//#define DHTTYPE DHT22   // DHT 22  (AM2302), AM2321

//#define DHTTYPE DHT21   // DHT 21 (AM2301)

// Connect pin 1 (on the left) of the sensor to +5V

// NOTE: If using a board with 3.3V logic like an Arduino Due connect pin 1

// to 3.3V instead of 5V!

// Connect pin 2 of the sensor to whatever your DHTPIN is

// Connect pin 4 (on the right) of the sensor to GROUND

// Connect a 10K resistor from pin 2 (data) to pin 1 (power) of the sensor

// Initialize DHT sensor.

// Note that older versions of this library took an optional third parameter to

// tweak the timings for faster processors.  This parameter is no longer needed

// as the current DHT reading algorithm adjusts itself to work on faster procs.

DHT dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(9600);

Serial.println(F("DHTxx test!"));

dht.begin();

}

void loop() {

// Wait a few seconds between measurements.

delay(2000);

// Reading temperature or humidity takes about 250 milliseconds!

// Sensor readings may also be up to 2 seconds 'old' (its a very slow sensor)

float h = dht.readHumidity();

// Read temperature as Celsius (the default)

float t = dht.readTemperature();

// Read temperature as Fahrenheit (isFahrenheit = true)

float f = dht.readTemperature(true);

// Check if any reads failed and exit early (to try again).

if (isnan(h) || isnan(t) || isnan(f)) {

Serial.println(F("Failed to read from DHT sensor!"));

return;

}

// Compute heat index in Fahrenheit (the default)

float hif = dht.computeHeatIndex(f, h);

// Compute heat index in Celsius (isFahreheit = false)

float hic = dht.computeHeatIndex(t, h, false);

Serial.print(F(" Humidity: "));

Serial.print(h);

Serial.print(F("%  Temperature: "));

Serial.print(t);

Serial.print(F("C "));

Serial.print(f);

Serial.print(F("F  Heat index: "));

Serial.print(hic);

Serial.print(F("C "));

Serial.print(hif);

Serial.println(F("F"));

}

- Demonstrate Ultrasonic distance sensor in arduino board.

Aim:

To find object distance detection using  Arduino board.

Procedure:

- Start the Arduino IDE and enter the code

- Declare the pin for echo and trigger

- Define the baud rate for serial transmission

- The loop function runs over and over again forever

- Based on the sensor output show the distance between the object and sensor

- Display the distance value in serial monitor.

const int trigPin = 9;

const int echoPin = 10;

// defines variables

long duration;

int distance;

void setup() {

pinMode(trigPin, OUTPUT); // Sets the trigPin as an Output

pinMode(echoPin, INPUT); // Sets the echoPin as an Input

Serial.begin(9600);  // put your setup code here, to run once:

}

void loop() {

// put your main code here, to run repeatedly:

digitalWrite(trigPin, LOW);

delayMicroseconds(2000);

// Sets the trigPin on HIGH state for 10 micro seconds

digitalWrite(trigPin, HIGH);

delayMicroseconds(2000);

digitalWrite(trigPin, LOW);

// Reads the echoPin, returns the sound wave travel time in microseconds

duration = pulseIn(echoPin, HIGH);

// Calculating the distance

distance= duration*0.034/2;

// Prints the distance on the Serial Monitor

Serial.print("Distance: ");

Serial.println(distance);

}

- Create an application to learn to test the output & input of GPIO pins on Raspberry pi.

Aim: To develop an application to learn the output and input operations of GPIO pins.

Procedure:

- Start the python environment in raspberry pi  and import the libraries

- Declare the variable for input pin

- Set pin 8 to be an output pin and set initial value to low (off)

- The while loop function runs over and turn on and off LED based on the output pin of Raspberry pi

- Turn the LED on by making the voltage HIGH

- Turn the LED on by making the voltage LOW

Program:

import RPi.GPIO as GPIO # Import Raspberry Pi GPIO library

from time import sleep # Import the sleep function from the time module

GPIO.setwarnings(False) # Ignore warning for now

GPIO.setmode(GPIO.BOARD) # Use physical pin numbering

GPIO.setup(8, GPIO.OUT, initial=GPIO.LOW) # Set pin 8 to be an output pin and set initial value to low (off)

while True: # Run forever

GPIO.output(8, GPIO.HIGH) # Turn on

sleep(1) # Sleep for 1 second

GPIO.output(8, GPIO.LOW) # Turn off

sleep(1) # Sleep for 1 second

- Develop an application to set up a Apache Tomcat web server on a Raspberry Pi.

sudo apt-get update

sudo apt-get install tomcat8

ifconfig	#find ip of rpi

e.g. 10.0.11.241

open browser on pc and type

10.0.0.11.241:8080

8080 is the port for tomcat

now in Rpi, type the following:

cd /var/lib/tomcat8/webapps

sudo mkdir iotlab #any name as you like

cd iotlab

sudo nano index.html

#now type the following code:

<html>

<head>

<title>Welcome to IoT Lab</title>

</head>

<body>

<H1>GOOD MORNING! WELCOME TO IOT LAB</H1>

</body>

</html>

now open the browser and type

10.0.11.241:8080/iotlab

- Design an prototype which measures the amount the gas present inside the working environment using gas sensor in Raspberry pi.

.

Aim: Develop  an application to measure the gas present inside the working environment

Procedure:

- Start the Arduino  IDE.

- Make the connection between gas sensor and Arduino

- The loop function runs over and over again forever

- Based on the sensor output show the current gas value.

- Display the gas values in serial monitor.

Program:

int LED1 = 12;

int LED2 = 11;

int buzzer = 10;

int smokeA0 = A0;

int sensorThreshold = 400;

void setup() {

pinMode(LED1, OUTPUT);

pinMode(LED2, OUTPUT);

pinMode(buzzer, OUTPUT);

pinMode(smokeA0, INPUT);

Serial.begin(9600);

}

void loop() {

int analogSensor = analogRead(smokeA0);

Serial.print("Pin A0: ");

Serial.println(analogSensor);

// Checks if it has reached the threshold value

if (analogSensor > sensorThreshold)

{

digitalWrite(LED1, HIGH);

digitalWrite(LED2, LOW);

tone(buzzer, 1000, 200);

}

else

{

digitalWrite(LED1, LOW);

digitalWrite(LED2, HIGH);

noTone(buzzer);

}

delay(1000);

}

8. Build the  application to monitor the environment using temperature sensor in Raspberry pi board.

Aim: Develop  an application to measure the environment temperature and push the information in thing speak cloud service

Procedure:

- Import the libraries in python module

- Make the connection between DHT sensor with Raspberry pi

- Specify the thing speak URL for posting the sensor data.

- Define the function to read the sensor data and compute the temperature and humidity values.

- Display the sensor data in serial monitor and also send it to thing speak cloud service.

Program

import sys

import Adafruit_DHT as dht

import urllib2

myAPI = 'IUKV2ZRBQW9MV407Q'

ThingsURL = 'https://api.thingspeak.com/update?api_key=%s' % myAPI

def DHT22_data():

humidity, temperature = dht.read_retry(dht.DHT22, 23)

return humi, temp

humidity, temp = DHT22_data()

if isinstance(humidity, float) and isinstance(temp, float):

humi = '%.2f' % humidity

temp = '%.2f' % temp

coms = urllib2.urlopen(ThingsURL + '&field1=%s&field2=%s' % (temp, humi))

print coms.read()

coms.close()

else:

print 'Error'

9.Implement smart cities Sensor Board v3.0 using Wasp mote platform for measuring temperature, humidity and pressure

Aim: To develop an application to measure temperature, humidity and pressure using Waspmote IDE.

Procedure:

- Include the waspmote header files

- Declare the temperature, humidity and pressure variables.

- Compile and upload the code in waspmote board

- Make the connection between waspmote board and sensor

- Print the values in serial monitor.

#include <WaspSensorCities_PRO.h>

/*

Waspmote OEM. Possibilities for this sensor:

- SOCKET_1

- SOCKET_2

- SOCKET_3

- SOCKET_4

- SOCKET_5

P&S! Possibilities for this sensor:

- SOCKET_A

- SOCKET_B

- SOCKET_C

- SOCKET_E

- SOCKET_F

*/

bmeCitiesSensor  bme(SOCKET_A);

float temperature;	// Stores the temperature in ºC

float humidity;		// Stores the realitve humidity in %RH

float pressure;		// Stores the pressure in Pa

void setup()

{

USB.println(F("Temperature, humidity and pressure sensor example"));

}

void loop()

{

///////////////////////////////////////////

// 1. Turn on the sensor

///////////////////////////////////////////

bme.ON();

///////////////////////////////////////////

// 2. Read sensors

///////////////////////////////////////////

temperature = bme.getTemperature();

humidity = bme.getHumidity();

pressure = bme.getPressure();

// And print the values via USB

USB.println(F("***************************************"));

USB.print(F("Temperature: "));

USB.printFloat(temperature, 2);

USB.println(F(" Celsius degrees"));

USB.print(F("RH: "));

USB.printFloat(humidity, 2);

USB.println(F(" %"));

USB.print(F("Pressure: "));

USB.printFloat(pressure, 2);

USB.println(F(" Pa"));

///////////////////////////////////////////

// 3. Turn off the sensor

///////////////////////////////////////////

bme.OFF();

///////////////////////////////////////////

// 4. Sleep

///////////////////////////////////////////

// Go to deepsleep

// After 10 seconds, Waspmote wakes up thanks to the RTC Alarm

USB.println(F("Go to deep sleep mode..."));

PWR.deepSleep("00:00:00:10", RTC_OFFSET, RTC_ALM1_MODE1, ALL_OFF);

USB.println(F("Wake up!!\r\n"));

}

10.  Measure the light intensity  and pollution level using Wasp mote platform.

Aim: To develop an application to measure the light intensity level using Waspmote IDE.

Procedure:

- Include the waspmote header files

- Declare the light intensity variable

- Compile and upload the code in waspmote board

- Make the connection between waspmote board and sensor

- Print the values in serial monitor.

Program

#include <WaspSensorEvent_v30.h>

// Variable to store the read value

uint32_t luxes = 0;

void setup()

{

// Turn on the USB and print a start message

USB.ON();

USB.println(F("Start program"));

// Turn on the sensor board

Events.ON();

}

void loop()

{

// Part 1: Read Values

// Read the luxes sensor

// Options:

//    - OUTDOOR

//    - INDOOR

luxes = Events.getLuxes(INDOOR);

// Part 2: USB printing

// Print values through the USB

USB.print(F("Luxes: "));

USB.print(luxes);

USB.println(F(" lux"));

delay(1000);

}